Q40: GATES

Project:

To be read with Preliminaries/General Conditions

Gate Type:		HFS CUSTOM GATES

Supplier:		Headstart Fencing (Supplies) Ltd

Headstart House, 39 High Street, Ware, Herts. SG12 9BA

Tel: 01920 468855 Fax: 01920 460044

Email: headstartfencing@aol.com								Contact:  Kevin Hill

- Double leaf and single leaf swing gates to HFS CUSTOM type general design.

Infill: 16mm solid bar

50x10 RSF Rails

101.6x10mm CHS 16mm long rings

Free Standing Gate Posts

- TBC – Arch Top, Finial Top, Convex, Full Arch

- TBC – Motif/Wording Optional

- TBC – Height/Width

- Locking System/Fittings

Gate to have a sliding latch for padlocking

Anti lift drop bolts

Adjustable hinges

- Finish

Galvanised to BS EN 1S0 1461

Galvanised & Polyester Powder Coated

- Installation

Setting Posts In Concrete

Gate to be set no more than 75mm above ground. Mix – BS5328, designated mix not less than GEN3 or standard mix not less than ST3 (alternative mix for small quantities – 50kg Portland cement class 42.5 to 120KG fine aggregate to 200kg, 20mm nominal maximum size coarse aggregate, medium workability).  Excavate holes neatly and with vertical sides.  Position post and fill hole with concrete to not less than the specified depth, well rammed as filling proceeds and consolidated.  Holes not completely filled with concrete to be backed-filled with excavated material, well rammed and consolidated.

Set out & erect gates in accordance with supplier approval drawing.

- COMPETENCE

- The contractor shall employ experienced installers and provide qualified supervision to complete works as specified by the manufacturer.